SOLID WASTE MANAGEMENT – COMPACT

TWIN SHAFT SHREDDER

| Nithya Sri R Department of Agriculture Engineering SNS College of Technology Coimbatore ,Tamilnadu nithyasriramesh1910@gmail.com | Harini SG Department of Agriculture Engineering SNS College of Technology Coimbatore ,Tamilnadu gopinathharini19@gmail.com | Yogesh D Department of Agriculture Engineering SNS College of Technology Coimbatore ,Tamilnadu yogeshdcs2002@gmail.com | Dr.B.Shridar Department of Agriculture Engineering SNS College of Technology Coimbatore ,Tamilnadu dean.agri@snsct.org |
| --- | --- | --- | --- |

Abstract - The key objective of the article is to design and build a small shredding machine that is conceptually designed with a focus on the production of dry leaves, areca leaves, chopped dry materials, and worm compost. This idea was produced by taking account of four slicing machines with various operations and operating procedures. The idea was designed with consideration for the customers' safety. A three-stage motor, bearings, cutter, structural frame, spur gear, and dual shafts make up the machine. High carbon steel, which is also used to make cutter tips and soft steel, is applied for making the machine's frame. Two shafts with sixteen cutters placed on them rotate parallel to the spur gear. The cutter shaft receives power from the electric motor through the belt drive. Tensile and friction inside the cutting house cause the cutting process. The chopped powder is gathered in a dish at the bottom after the dried leaves are chopped and reduced in size. It is simple to use and has a small footprint.

Keywords: Hexagonal Shaft, Blade, Spacer, Spur Gear, Shredding Machine, Agro Waste

# Introduction

## In many regions of the world, agriculture is the most intense business, producing a wide range of waste water that needs to be treated and managed using several approaches. In India, the principal occupation accounts for 70% of the population. In India, many different crops are grown, but after harvest, crop wastes are burned or discarded without consideration for their nutritional worth. Sustainable agriculture not only balances population increase, but it also compels us to grow food in a sustainable way.

## Pesticides and fertilizers used in excess will have an adverse effect on soil health, lower crop yields, and lower product quality. In order to preserve health and property, the natural equilibrium must be kept at all costs.

Agro waste, also known as agricultural waste or crop residue, refers to the by-products or leftovers from agricultural activities. It includes various organic materials that are generated during the cultivation, processing, and utilization of crops and animals in agriculture.

Agro waste can come in various forms, such as crop stalks, husks, shells, leaves, stems, roots, straw, chaff, and animal manure. These materials are typically considered as waste because they are not directly used for food or feed purposes and are often left unused or burned in the field, leading to environmental and health issues.

However, agro waste can also have potential uses and benefits. It can be utilized for various purposes, such as bioenergy production, animal feed, composting, soil enrichment, and production of bio-based materials. For example, crop residues can be converted into biofuels, such as bioethanol or biomass pellets, through processes like fermentation or pyrolysis. Animal manure can be used as fertilizer or converted into biogas through anaerobic digestion. Crop residues and agro waste can also be used to improve soil fertility and structure, reduce erosion, and support sustainable agricultural practices.

The core objective of the paper is to create a shredding machine that can produce tiny pieces of vegetables, leaves, and coconut leaves and turn them into fertiliser for crops. The hopper, spur gear, dual shaft, frame, motor, cutter, belt, and coverings are all part of this machine. The machine is constructed utilising a tungsten carbide cutter and the cutting point of a light steel frame. A 10-HP power supply with 1 stage powers the device. The hopper of this machine is vertically inserted with leaves of coconut powder. The motor rotates at a predetermined speed of 1440 rpm. Maintain a about 120:1 speed ratio such that the belt and the shaft are driven at 520 revolutions per minute. Coconut leaves and vegetable waste are hacked and dried later for use as manure due to its strong rotating force.

## EXISTING METHODOLOGY

The paper was made from bits of dried coconut leaves, which were then composted. The disposal of traditional and ancient trash, in which agricultural waste is degraded, is known as traditional agro-waste disposal. Waste is thrown away; thus, it takes longer to disintegrate and as a result, it could harm the ecosystem. By using the crumble machine, less agricultural waste will be needed as fertilizer. The cutter, mounted on the dual shaft, fixed to the motor base, and the V-belt, which is supported by a large pulley connected to the small pulley gear at the end of the motor, are the key components of a little piece of machinery, according to the knowledge acquired about it. The barrel revolves in the transmission because one gear drives the other gear.

To establish the experimental relationship for a motorized chaff cutter powered by a flywheel motor powered by a human, the authors of developed an experimental setup design in. The main purpose of this machine is to chop up food so that animals can consume it conveniently. Cycling is suggested as a means of converting and transmitting human force into the rotational dynamics of a flywheel by paddling in the concept of a man-powered flywheel motor. The flywheel's stored energy is utilized during the cutting operation. It was discovered that the driver used a gear ratio of 1: 2 for the paddle and the flywheel shaft to maintain a speed of 350 RPM for 1 minute. After one minute, the paddling ceased, and the experimental setup's free operation was confirmed. The flywheel shaft rested after 25 minutes. This demonstrates that the alignment of the bearing with the other test setup elements results in satisfactory performance.

Organic waste is fed to drums and trays equally in their research on the Methodology for Design and Fabrication of Portable Organic Waste Chopping Machine. An electric motor then rotates the shaft through the pulley at 1440 revolutions per minute, pushing the cutting drum to shear the organic wastes with the help of the shearing blades. Because of the tensile, friction, and impact effects of the cutting process, the chopping is also done inside the cutting house. After passing through the sieve's dipped holes with the chopped pieces of organic waste, the machine releases them. Different sized holes can be used. The construction of the machine for extracting areca fibre was the authors' main emphasis in.

Its main job is to take the fibre out of the areca. A straight shaft and a 3 phase, 5 HP AC motor are attached to the machine. An enclosure casing is joined to the driving shaft. The rectangular duct at the bottom of the fibre casing is where the casing emerges. It is designed to solely remove dust. Two bearings and blades that were modified from the blades of the machine used to extract coconut fibre support the driving shaft. The machine produces good-quality areca fibre with a diameter of 0.39 to 0.12 mm and a length of 5 to 6 cm. Therefore, farmers and rural business owners’ profit from this fibre extraction machine.

A machine for extracting coconut fibre that can be used by small-scale coir manufacturing enterprises was invented and developed by authors in. With the aid of a V-belt, the drive shaft is connected to the engine in this model at the other end of the drive shaft, where the drive shaft is coupled to a heavy duty, one-phase AC motor at the base. This allows the barrel to rotate at 240 rpm. The spherical coconut shell mechanically slides to the opposite end of the barrel as the coconut is fed from one end to the other, and the separated fibre is collected in the bag provided below. The indicator hole on the surface of the barrel is where the cutting pins are attached in this concept. removing the fibre by cutting the pin supports To establish the experimental relationship for a motorised chaff cutter powered by a flywheel motor powered by a human, the authors of developed an experimental setup design. The main purpose of this machine is to chop up food so that animals can consume it conveniently. Cycling is the idea of a man-powered flywheel motor.

Paddling was suggested as a way to transform and transmit human power into the rotating dynamics of the flywheel. The flywheel's stored energy is utilized during the cutting operation. It was discovered that the driver used a gear ratio of 1: 2 for the paddle and the flywheel to reach a speed of 350 RPM. The indicator hole on the surface of the barrel is where the cutting pins are attached in this concept. removing the fibre by cutting the pin supports To establish the experimental relationship for a motorised chaff cutter powered by a flywheel motor powered by a human, the authors of developed an experimental setup design in. This device is. After one minute, the paddling ceased, and the experimental setup's free operation was confirmed. The flywheel shaft rested after 25 minutes. This demonstrates that the alignment of the bearing with the other test setup elements results in satisfactory performance.

Organic waste is fed to drums and trays equally in their research on the Methodology for Design and Fabrication of Portable Organic Waste Chopping Machine. After that, the shaft is turned through the cutting drum to chop the organic wastes with the impact of shear generated by the shearing blades by turning the pulley at 1440 revolutions per minute (rpm). Because of the tensile, friction, and impact effects of the cutting process, the chopping is also done inside the cutting house. After passing through the sieve's dipped holes with the chopped pieces of organic waste, the machine releases them. Different sized holes can be used. The construction of the machine for extracting areca fibre was the authors' main emphasis in. Its main job is to take the fibre out of the areca. A straight shaft and a 3 phase, 5 HP AC motor are attached to the machine. An enclosure casing is joined to the driving shaft.

The rectangular duct at the bottom of the fibre casing is where the casing emerges. It is designed to solely remove dust. Two bearings and blades that were modified from the blades of the machine used to extract coconut fibre support the driving shaft. The machine produces good-quality areca fibre with a diameter of 0.39 to 0.12 mm and a length of 5 to 6 cm. Therefore, farmers and rural business owners’ profit from this fibre extraction machine.

A machine for extracting coconut fibre that can be used by small-scale coir manufacturing enterprises was invented and developed by authors in. With a little cover at the base, the engine is connected to a heavy-duty, one-phase AC motor in this type a V-belt connects the drive shaft at the opposite end of the drive to the motor's drive shaft, allowing one gear to drive the other gear while the barrel rotates at 240 revolutions per minute. The spherical coconut shell mechanically slides to the opposite end of the barrel as the coconut is fed from one end to the other, and the separated fiber is collected in the bag provided below. The indicator hole on the surface of the barrel is where the cutting pins are attached in this concept removing the fiber by cutting the pin supports to establish the experimental relationship for a motorized chaff cutter powered by a flywheel motor powered by a human, the authors of developed an experimental setup design.

The indicator hole on the surface of the barrel is where the cutting pins are attached in this concept. Cutting the supports for the pins will allow the fibre to be extracted and will provide the fibre with a linear path to leave the coconut shell. Cutting pin indexing angle and space between them are crucial considerations in the construction of the machine that extracts coconut fibre.

They created a concept for a machine for crushing trash in. The waste crushing machine can be employed as a ploughing attachment-style device. The shredder will be run by a tractor's power take-off shaft (PTO). Five circular blades and one fixed blade make up this machine. They cut up organic material into tiny pieces so the farmer could use it to make vermin compost.

This work's scope includes conceptualizing and the indicator hole on the surface of the barrel is where the cutting pins are attached in this concept. Cutting the supports for the pins will allow the fibre to be extracted and will provide the fibre with a linear path to leave the coconut shell. A shredder machine focused on producing dry leaves, areca leaves, thus chopped dry, vermin compost, was developed in the design of coconut fibre end. They base their work on information and data gathering about consumer lifestyles. Safety Aspect The concept was created with the management of customers and the working environment in mind.

In this section a gear and pulley mechanism with a belt drive is employed. The solid wastes that are too inflexible to digest and degrade are broken down into manageable pieces by this machine. The agricultural waste can break down and be fed to domestic animals and livestock as a wholesome diet.

Plastics are one of the most significant technical tools utilized in a variety of applications. Cutting plastic materials to the appropriate size is one of the crucial factors to consider while processing them. Both a handsaw and an automobile machine can be used for this cutting. The shredder machine is frequently used to separate garbage and the production's basic plastic for further processing. Thrown-in pollutants have an impact on the ecosystem and produce pollution. Reducing waste from plastic recycling is a crucial task. However, one of the issues with recycling these waste goods is the multi-bladed and compressed by shredder size machine.

Therefore, the aim of this project is to create a twin shafted, multibladed shredding machine that runs on electricity and uses local machinery to recycle waste plastic for future use. This study was carried out using the experimental methodology needed for variable and parameter control. The sample was 1227x854x700 mm in size and was drawn using a purposeful sampling technique. The required factors include the blade's angle, sharpness, spacing, etc., while some criteria include cutting speed and revolutions per minute. Small portions have been constructed using common production techniques, such as machinery and measurement instruments.

They must package this garbage and deliver it to nearby processing facilities. So, the cost of shipping and packaging is also increased. As a result, the authors try to make processing plastic waste as inexpensive as possible in order to cut labor costs. A cutting device is used to reduce the size or number of fragments of big solids. The introduction of the plastic bottle cutting machine and an investigation of the mechanical system are both covered in this project. A machine called a plastic bottle cutter is used to shred plastic into tiny bits for garbage disposal. The authors created a cutting machine prototype for home plastic trash recycling. This device is beneficial for both industries and Scrap Collectors.

## PROPOSED METHODOLOGY

DESIGN CALCULATIONS

To determine the mass of compartment

Material choice for compartment = Mild Steel.

Properties of Mild Steel to be used:

Density = 7850 kg/m3

Young’s Modulus = 2.08×105 N/mm2

Poisson’s Ratio = 0.3

3D Image of Shredder Machine

Mass = Density * volume

Volume = length *breadth * height

= (0.35*0.32*0.32) Volume =0.03584 m3

Mass     = 7850 * 0.03584

= 281.34 kg

Mass of compartment = 281.34 kg

Shaft calculations

Hexagonal shaft is chosen

Weight of shaft = A/F * A/F * length of shaft* 0.0078

Across face (A/F) = 36 mm

Length of shaft = 400mm

Weight of shaft = 0.036 *0.036 *0.4 *0.0078

Weight of shaft = 3.52 kg

Torque = Force * perpendicular distance

Force   = Weight * center of gravity

= 15.67 * 9.81

Force (F) = 153.72 N

Torque    = 153.72 * 0.4

= 61.488 Nm

Torque transmitted by shaft = 61.488*103 Nmm

Material of shaft is mild steel

Yield strength (σy) = 250N/mm2

Designed shear stress(τ) = τy / FOS

τy = 0.5 * σy

= 0.5 * 250 τy

= 125 N/mm2 τ

= 125 / FOS FOS

= 3 τ = 125 / 3 τ = 50 N/mm2

Across face factor of shaft is 36 mm

Blades and spacer calculation

Thickness of blade= 10 mm

Weight of one blade = 0.75 kg

Weight of one spacer = 0.2 kg

For one shaft 12 blades and 13 spacers

So total weight= [(12*0.75) + (13*0.2)]

Total weight = 12 kg

So

Total weight of shaft with blades and spacer is 15.52 kg

Selection of motor

Weight of shaft = 3.52 kg

Length of shaft = 400mm

Blade and spacer for shaft:

Thickness of blade= 10 mm

Weight of one blade = 0.75 kg

Weight of one spacer = 0.2 kg

For one shaft 12 blades and 13 spacers

So total weight= [(12*0.75) +(13*0.2)]

Total weight = 12 kg

So,

Total weight of shaft with blades and spacer is 15.52 kg Force acting = 153.72 N

Torque = 61.48 Nm (for single shaft)

For both shafts:

T= 61.488 * 2

T= 122.976 Nm (for no load)

With load,

T = 500 Nm (allowable)

P = 2ΠNT / 60

T = P * 60 / 2ΠN

= [((2.2*103) * 60) / (2*Π* 48)]

Torque = 437.89 Nm

3HP motor is selected

Selection of gearbox

We need to shred approximately 300 kg of wastes per hour

Which means 8.3 kg/min

Gearbox ratio = 1:30

1440: 48

If 48 revolutions per minute then for a second 0.8 revolutions

0.8/ 8.3 =0.096 kg/s

0.096* 60 = 5.78 kg/min

5.78 * 60 = 346.98 kg

The output per hour will be around 346 kg

1:30 ratio gearbox is chosen

Bearing calculations

Diameter of the shaft = 30 mm

Deep groove ball bearing is selected.

Specification of selected Bearing:

d= 30mm

D= 37mm

Approx. life hours = 1440 hrs

(Co) Standard capacity = 14600N

(C) Dynamic capacity   = 22000N

𝑃 = (𝑋 ∗ 𝐹 𝑟 ) + (𝑌 ∗ 𝐹 𝑎 )

F r = radial force                 F a = axial force

𝐹𝑎 /𝐶𝑜 = 0.102

𝐹𝑎 / 𝐹𝑟 = 1.93

Service factor(s) = 1.5 for rotatory

Fa/Fr >e

e = 0.31

x = 0.56                               y= 1.4

𝑃 = ((𝑋 ∗ 𝐹 𝑟 ) + (𝑌 ∗ 𝐹 𝑎 )) ∗ 𝑆

= ((0.56 * 2900) + (1.4 * 1500)) * 1.5

P = 5586 N

Check bearing capacity from the graph

𝐶/P = 2.12

C / 5586 = 2.12

C = 5586* 2.12

C = 11842.32 N

𝐶 𝑐 𝑎 𝑙 𝑐 𝑢𝑙𝑎𝑡𝑒𝑑 < 𝐶 𝑑 𝑦 𝑛 𝑎𝑚𝑖𝑐

11842.32 < 22000 N

Design is safe

Gear calculations

Deep groove ball bearing SKF 63062Z bearing is selected. Center distance (a)= 125 mm

Z1= 22 teeth

Z2 = 27 teeth

To find module (m):

m = [2a] / [Z1+Z2]

= [2*125] / [22+27]

= [250] / [49]

m = 5.10

Module(m)=5

Diameter of drive and driven gear (d) =m*z

d1 = m*Z1

= 5.10 * 22

d1 =112.2 mm

d2 = m*Z2

= 5.10 * 27

d2 =137.7 mm

i    = Z2 / Z1

= 27/22

i = 1.22

Form factor(y) = 0.124 – 0.684/z (for 14o)

= 0.124 – 0.684/22

Y= 0.031

= 0.124 -0.684/27

Y =0.02533

Dynamic load (Fd) = Ft + 0.164Vm*c+ Ft 0.164Vm + 1.485 C+Ft Ft = Hp * 75/Vm

Mean velocity (Vm) = 10 for commercial cut gears

= 3* 75/10 Ft

= 22.5

Expected error =0.056 (for module= 5)

Fd = 22.5 + 0.164*400 (439.6+22.5) 0.164*400 + 1.485 22.5

Dynamic load (Fd) = 417.31 N

Fd calculated value = 417.31N

Fd Maximum = 800 N

Fd calculated < Fd maximum

Design is safe

To Find whether the frame Design is Safe/Not

RA =Reaction at A

RB = Reaction at A

RA = RB = (W*C)/2

= (3*0.8)/ 2

= 1.2 KN

Fx = RA – W*X = (W*L)/2 – W*X

Shear force:

At A X=0

FA = [(W*L)/2] – [(W*X)/2]

= (3*0.8)/2 - (3*0)/2

= 1.2 – 0 FA

= 1.2 KN

At B X=L

Hence FB = [(W*L)/2] – W*B

= (3*0.8)/2 – 3*0.8

= 1.2 – 2.4

FB = -1.2 KN

At C X=L/2

Hence FC = [(W*L)/2] – [(W*X)/2]

= (3*0.8)/2 - (3*0.8)/2

= 0 KN

Bending moment:

At A X=0

Hence MA = [(W*L)/2]* X * [(W*X)/2]

= (3*0.8)/2 * 0 * (3*0.8)/2

= 0 KNm

At B X=L

Hence MB = [(W*L)/2]* L – W/2 *L2 = (3*0.8)/2 * 0.8 – 3/2 * (0.82 ) = 0 KNm

At C X=L/2

Hence MC = [(W*L)/2] * L/2 – [W/2 * (X2 )]

= (3*0.8)/2 * 0.8/2 - (3/2) * (0.8/2)2

= 1.2 * 0.4 – 1.5*0.16

MC = 0.24 KNm

To Find whether the shaft

Design is Safe/Not

RA + RB = 29 N

Shear force:

(29*200) * (RB *400) = 0

RB = 5800/400

= 14.5 N

Bending moment: M= (29*200)

= 5800 Nmm

Key and keyway calculation

Diameter of shaft (d) = 30 mm

Key cross section:

Breadth = 10 mm

Height  = 8 mm

In shaft = 5 mm (Tolerance +0.2)

In gear = 3.3 mm (Tolerance +0.1)

Length (L) = 40 mm (from design data book)

Shear stress in key:

Mt = T= L * W * τK *(d/2)

P = 2πnMt/60

Mt = P*60/2Πn

= [(2.2*103)*60] / [2*π*48]

= 437.67 Nm Mt

= 437.67*103 Nmm

Mt = T= L * W * τK *(d/2)

437.67*103 = 40*10* τK * (30/2)

τK = 72.945 N/mm2

σy = 330 N/mm2 at C40

τy = σy / 2

= 330/2

Yield shear stress (τy) = 165 N/mm2

τK < τy

72.945 N/mm2 < 165 N/mm2

Design is safe

Selection of bolt

Bolt material is mild steel

Shear stress of mild steel (τ) = 250 N/mm2

Yield stress (τy) = 250 / 2

= 125 N/mm2

Designed shear stress = τy/ FOS

= 125/3

= 41.6 N/mm2

Load acting on single bolt is 17 N 
                        σ = PS / A

41.6 = 17/A

Area = 0.408 mm2

Based on the area value bolt will be chosen (from design data book)

5/8” bolt is chosen

Calculation of welding fillet

σ = P/(h*l)

P =200 Kg (200*9.81)

= 1962 N

To find stress acting on plate:

σ = P/A

= 1962/17500

Stress acting on plate (σ) = 0.95 N/mm2

Applying stress value in welding formula

0.95 = 1962/ (h*l)

Welding length = 350 mm

h = 1962/ (0.95*350)

Size of weld (h) = 5.9 mm

DESIGN OF TWIN SHAFT SHREDDER MACHINE

## CONCLUSION

The major goal of this article is to build and develop a small shredding machine that can chop up vegetables, leaves, and coconut leaves into tiny bits and use those pieces as fertilizer. The public and private sectors can both employ this small cutting device. Every day, waste is gathered from homes and flats in the public sector and crushed into equipment. After being combined with coconut pits for a few days, the substance is eventually transformed into organic fertilizer. The compact shredding machine's design and construction for the on-site composition Centre were given in this study. A model was created, and the machine's efficiency was judged to be good.

## REFERENCE

[1] I M Sanjay Kumar “Design and Fabrication of Coconut Leaves Shredder. International Journal Of Engineering Research And General Science”. , Pp273-281, 2015.

[2] Dr T R Hemanth Kumar “Design and development of Agricultural Waste Shredder Machine”, International Journal of Innovative Science, Engineering and Technology. 2(10), Pp164-172, 2014.

[3] P.B.Khope and J.P.Modak “Design of experimental set-up for establishing empirical relationship for chaff cutter energized by human powered flywheel motor” Journal of Agricultural Technology 2013 Vol. 9(4): 779-791.

[4] Ajinkya S.Hande, Manuel A.Bajet. “Methodology For Design & Fabrication of Portable Organic Waste Chopping Machine To Obtain Compost -A Review” IJIRST –International Journal for Innovative Research in Science & Technology| Volume 1 | Issue 7 | December 2014 ISSN (online): 2349-6010

[5] Krishna Naik, Mohamed khatib iqbal. “Design and fabrication of Areca fibre extraction Machine” International Journal of Emerging Technology and Advanced Engineering ISSN 2250-2459, ISO 9001:2008 Certified Journal, Volume 4, Issue 7, July 2014

[6] Y. Prashanth, Gururaja. “Design and Development of Coconut Fiber Extraction Machine” Department of Design, M. S. Ramaiah School of Advanced Studies, Bangalore - 560 058 Volume 13, Issue 1, April 2014.

[7] S.Nithyananth, Libin Samuel “ Design of Waste Shredder Machine” Int. Journal of Engineering Research and Applications ISSN : 2248-9622, Vol. 4, Issue 3( Version 1), March 2014, pp.487-491.

[8] Sreenivas H T. “Conceptual Design And Development Of Shredding Machine For Agricultural Waste”, International Journal Of Innovative Research In Science, Engineering And Technology. , Pp 7317-7323, 2016.

[9] Lee, J. -. (2019). Examination of stress intensity factors for composite materials by FeM (II). Test Engineering and Management, 81(11-12), 204-210.