ENHANCING SEISMIC ANALYSIS OF BRIDGES THROUGH SOIL-STRUCTURE INTERACTION

Vishwas P Ga, Ashwini Satyanarayanab1

a Post graduate Student, Department of Civil Engineering, Dayananda Sagar College of Engineering, India,

E-mail: vishwaspg06@gmail.com

b1Assistant Professor, Department of Civil Engineering, Dayananda Sagar College of Engineering, India,

E-mail: ashwini-cvl@dayanandasagar.edu

ABSTRACT — Seismically isolated bridge structures are widely employed as a crucial measure for mitigating the destructive effects of earthquakes. The effectiveness of these isolation systems relies not only on the properties of the bridge structure but also on the interaction between the structure and the underlying soil. The understanding of soil-structure interaction (SSI) plays a vital role in designing and assessing the performance of seismically isolated bridges. This project aims to investigate the influence of SSI on seismically isolated bridge structures by considering variations in soil types under different ground motion scenarios.

To achieve this objective, the study will employ numerical modelling techniques and advanced analysis tools. First, a comprehensive literature review will be conducted to identify and summarize the key parameters affecting SSI in seismically isolated bridges. These parameters include soil stiffness, damping characteristics, and nonlinear soil behaviour. Subsequently, a numerical model representing a seismically isolated bridge system will be developed, considering different soil types commonly encountered in bridge engineering, such as sandy soil, clayey soil, and layered soil profiles.

The project will consider a range of ground motion records with varying intensity levels and frequency content. The ground motions will be applied to the bridge system model, incorporating the selected soil types. The response of the bridge structure, including displacements, accelerations, and internal forces, will be evaluated for each combination of soil type and ground motion. Comparative analyses will be performed to assess the variations in the dynamic response of the seismically isolated bridge due to different soil types.

The findings of this research project will provide valuable insights into the significance of soil-structure interaction on the performance of seismically isolated bridges. The results will enhance the understanding of the variations in soil types which can affect the structural response of seismically isolated bridges under different ground motions. The outcomes will be beneficial for bridge engineers and researchers involved in the design, evaluation, and retrofitting of seismically isolated bridge structures, facilitating the development of more accurate design guidelines and improved assessment methodologies.

Keywords — Soil-structure interaction, Ground motions, Numerical modelling, Dynamic response

INTRODUCTION:

Bridges serve as essential components of modern infrastructure, connecting communities, facilitating trade, and enabling efficient transportation. However, these critical structures are vulnerable to seismic hazards, posing a substantial risk to their integrity and functionality. To ensure the resilience and safety of bridges in regions prone to earthquakes, it is imperative to employ advanced engineering techniques that consider the intricate interplay between the bridge's superstructure and the underlying soil. This intricate interaction, known as Soil-Structure Interaction (SSI), is pivotal in determining the bridge's response to seismic forces and warrants comprehensive exploration and understanding.

Historically, seismic analysis of bridges has predominantly centered on the bridge's superstructure, often neglecting the influence of the surrounding soil. However, recent advancements in structural engineering and geotechnical research underscore the significance of incorporating SSI effects into seismic analysis. By accounting for the dynamic interaction between the bridge and the supporting soil, engineers can develop more precise models that reflect the realistic behavior of bridges during seismic events. This approach enables the enhancement of seismic analysis and the formulation of more resilient bridge designs capable of withstanding severe seismic disturbances.

This paper aims to delve into the pivotal role of Soil-Structure Interaction in elevating the seismic analysis of bridges. It will scrutinize various factors that impact this interaction, encompassing soil characteristics, bridge design attributes, and seismic ground motion characteristics. Additionally, this paper will illuminate the advantages stemming from the inclusion of SSI in bridge analysis, including heightened accuracy in predicting structural responses, improved comprehension of load transfer mechanisms, and the potential for optimizing bridge design and retrofitting strategies.

The discussion will also encompass cutting-edge computational tools and modeling techniques that have emerged in recent years to faithfully simulate SSI. These tools empower engineers to conduct more sophisticated seismic analyses that consider not only the dynamic response of the bridge superstructure but also the underlying soil, empowering them to make well-informed decisions regarding bridge design, retrofitting, and maintenance.

Furthermore, this paper will explore tangible applications and case studies where SSI analysis has been pivotal in enhancing the seismic resilience of bridges. These real-world examples will underscore the tangible benefits of factoring SSI considerations into bridge engineering, spanning from heightened safety for bridge users to reduced economic losses during seismic events.

In conclusion, comprehending and incorporating Soil-Structure Interaction is paramount for advancing the seismic analysis of bridges. By recognizing the intricate interplay between a bridge and its foundation soil, engineers can craft more precise models, optimize bridge designs, and, ultimately, assure the safety and resilience of these critical infrastructure elements in earthquake-prone regions. This paper will offer valuable insights into the importance of SSI and its practical implications for the realm of bridge engineering and the mitigation of seismic risks.

The existing bridge, situated in Beed along National Highway No. 211, spans over the Bindusara River. This bridge boasts dimensions of 42 meters in length and 7.95 meters in width, featuring a structure comprised of three continuous reinforced concrete girders. The superstructure, on the other hand, is constructed with a 300 mm deep reinforced concrete deck slab, which is supported by the three reinforced concrete girders, each with a depth of 1100 mm. The bridge's solid piers, measuring 4.0 meters in width and 1.0 meter in thickness, rest securely on a stable soil stratum. In the bridge modeling process, the deck is represented using thin shell elements, while the girders and piers are modeled as frame elements. For further accuracy, elastomeric bearings are modeled as linear-type links, and elastomeric isolators are represented as rubber isolator-type link elements.

Top of Form

Figure 1 illustrates the bridge model analyzed using CSI bridge.

In the present study, our initial focus centers on modeling and analyzing the first bridge employing conventional elastomeric bridge bearings. Subsequently, we gather and examine the results obtained from this analysis. Following this, we undertake a pivotal step by substituting these conventional elastomeric bearings with elastomeric isolation bearings, conducting a comprehensive analysis, and subsequently comparing the results.

The fundamental distinction between conventional elastomeric bearings and elastomeric isolation bearings lies in their horizontal flexibility, with the latter being notably more flexible in the horizontal direction while maintaining the same vertical stiffness. This increased horizontal flexibility, combined with specific damping characteristics, serves as the foundation for achieving the desired isolation effects within the system.

The enhanced horizontal flexibility of the elastomeric isolation bearings acts as a mechanism that restricts the transmission of substantial earthquake forces from the piers to the superstructure. Conversely, the damping properties of these bearings play a crucial role in dissipating seismic energy, thereby effectively reducing the overall design displacement experienced by the bridge.

For the earthquake analysis of the isolated bridges under consideration, several key assumptions have been made to guide our study and analysis.Top of Form

For our earthquake analysis of the isolated bridges under consideration, several key assumptions have been made to guide our study:

- Elastic Superstructure and Piers: It is assumed that both the bridge superstructure and piers remain within the elastic state during earthquake excitation. This assumption is well-founded, as the isolation system's primary objective is to reduce earthquake response in a manner that keeps the structure operating within its elastic range.

- Straight Bridge Deck: The bridge deck is assumed to be straight, and both the deck and abutments of the bridge are considered rigid in our analysis.

- Rigidly Fixed Piers: The bridge piers are assumed to be rigidly fixed at the foundation level, implying minimal movement at the connection between the piers and the foundation during seismic events.

- Stable Foundation: The bridge is founded on stable soil or rock, and any potential soil-structure interaction effects are disregarded in this analysis. This simplifying assumption allows us to focus on the impact of the isolation system.

- Uniform Bearing Properties: The bearings provided at both the abutments and piers are considered to have the same dynamic properties. This simplification streamlines our analysis by assuming uniform behavior across these bearing elements.

These assumptions serve as the basis for our earthquake analysis, providing a framework for our study of isolated bridges and their response to seismic events.

In our seismic analysis, we employ a rigorous methodology that begins with the selection of the Imperial Valley earthquake ground motion record to replicate real-world seismic conditions. We then construct a detailed and accurate model of the bridge, accounting for its various components, including the superstructure, piers, abutments, and bearings. Initially, the bridge is subjected to analysis with conventional elastomeric bridge bearings, providing us with a baseline understanding of its seismic response within the constraints of traditional bearing systems. Subsequently, we replace these conventional bearings with elastomeric isolators, each with carefully defined stiffness properties. These properties are pivotal in determining how the bridge behaves under seismic forces. By systematically following this approach, we can perform a comparative assessment of the bridge's seismic performance, evaluating the effectiveness of elastomeric isolation in enhancing its resilience and reducing its vulnerability to seismic events.

METHEDOLOGY

This project employs a comprehensive methodology to investigate the effects of soil-structure interaction on the seismic response of bridges. Beginning with an in-depth literature review, the study establishes a foundation by examining existing theories and methodologies in this field. Clear objectives are then defined to address specific aspects, including the impact of different soil types, bridge configurations, and seismic loading conditions. Representative bridge models are carefully selected, encompassing various types, materials, and geometries to ensure the study's applicability to real-world scenarios. Soil characterization involves rigorous testing to determine key properties such as shear modulus and damping ratio, essential for accurate modeling. Advanced numerical simulations, utilizing finite element analysis software, are employed to model the selected bridges and simulate their behavior under seismic loading, with a particular emphasis on incorporating realistic soil-structure interaction effects. Parametric studies explore the sensitivity of the system to variations in soil stiffness, foundation type, and bridge geometry. Complementing these simulations, scaled physical models of the bridges undergo experimental testing, providing validation and capturing insights that may elude numerical analyses alone. Data from both approaches are meticulously analyzed to identify trends, correlations, and critical factors influencing the seismic response. The study's conclusions not only contribute to a deeper understanding of soil-structure interaction but also offer valuable insights for the refinement of seismic design codes, recommending considerations for practitioners and potential improvements to existing standards. The entire process is documented comprehensively in a report, ensuring that the methodology, results, and implications are communicated effectively within the field of earthquake engineering

Top of Form

TABLE 1. Stiffness values of Elastomeric bearing and its variations

|  | KV (kN/m) | KH (kN/m) | KL (kN/m) |
| --- | --- | --- | --- |
| Elastomeric Bearing | 457000 | 7500 | 7500 |
| 25% increase in Elastomeric bearing | 571250 | 9375 | 9375 |
| 25% decrease in Elastomeric Bearing | 342750 | 5625 | 5625 |
| 50% increase in Elastomeric Bearing | 685500 | 11250 | 11250 |
| 50% decrease in Elastomeric Bearing | 228500 | 3750 | 3750 |

# INPUT GROUND MOTION

Accelarations(mm/sec^2)

Time (sec)

Ground acceleration Vs. Time record of Imperial Valley earthquake

Ground motion records of 180° component recorded at ELCentro during May 18, 1940 Imperial Valley earthquake were obtained and used in study

RESULTS AND DISCUSSION

After performing time history analysis of bridge, seismic response of bridge is scrutinized and compiled results are presented in following TableTop of Form

TABLE 3: Results of TIME PERIOD

TABLE 4: Results of SHEAR

TABLE 5: Results of ABSOLUTE ACCELERATION

TABLE 6: Results of DISPLACEMENT

In above Table  Significant increase in fundamental time period,acceleration and shear while decrease in the displacements is observed due to horizontal flexibility of elastomeric isolator and resultant change in seismic response is reduction in absolute acceleration of deck and base shear in pier values.

CONCLUSION

In conclusion, the investigation into the effects of soil-structure interaction on the seismic response of bridges has provided valuable insights into the dynamic behavior of these structures under earthquake loading. Through a combination of numerical simulations and experimental testing of scaled models, the project has shed light on the significance of considering soil-structure interaction in seismic design.

The results highlight the intricate relationship between the bridge and underlying soil conditions, emphasizing the need for accurate modeling in seismic analysis. The observed effects on structural response and performance underscore the importance of incorporating soil-structure interaction considerations in the design process. The findings contribute to advancing our understanding of bridge behavior during earthquakes and have implications for the development of more robust and reliable seismic design codes for bridges.

As a result of this project, engineers and researchers are better equipped to make informed decisions in the seismic design of bridges, taking into account the nuanced interactions between the structure and its foundation. This knowledge can ultimately enhance the resilience of bridges, ensuring their ability to withstand and effectively mitigate the impact of seismic events.

REFERENCES

- Skokandi ́c, D.; Vlaši ́c, A.; Kušter Mari ́c, M.; Srbi ́c, M.; Mandi ́c Ivankovi ́c, A. Seismic Assessment and Retrofitting of Existing Road Bridges: State of the Art Review. Materials 2022, 15, 2523. https:// doi.org/10.3390/ma15072523

- Shawn Parks, Mijia Yang, Sivapalan Gajan and Qiang Pei,"Strength-based differential tolerable settlement limits of bridges",Advances in Structural Engineering 1–13 The Author(s) 2017  DOI: 10.1177/1369433217706779

- Alessandra De Angelis, Michele Mucciacciaro, Maria Rosaria Pecce and Stefania Sica,"Influence of SSI on the Stiffness of Bridge Systems Founded on Caissons",analyses. DOI: 10.1061/(ASCE)BE.1943-5592.0001073. © 2017 American Society of Civil Engineers.

- Murat DICLELI , Jung -Yoon LEE and Mohamad Mansour,"IMPORTANCE OF SOIL-BRIDGE INTERACTION MODELING IN SEISMIC ANALYSIS OF SEISMIC-ISOLATED BRIDGES",13th World Conference on Earthquake Engineering Vancouver, B.C., Canada August 1-6, 2004 Paper No. 3147

- Shinozuka, M., Kim, SH., Kushiyama, S. et al. Fragility curves of concrete bridges retrofitted by column jacketing. Earthq. Engin. Engin. Vib. 1, 195–205 (2002). https://doi.org/10.1007/s11803-002-0065-2

- Faisal Mehraj Wani,Jayaprakash Vemuri ,Chenna Rajaram ,Dushyanth V Babu R, Effect of Soil Structure Interaction on the Dynamic Response of Reinforced Concrete Structures, (NHRES 60)-2022

- Sopna S Nair, Dr. G Hemalatha, Dr. P Muthupriya, Vulnerability Assessment Using Fragility CurvesInternational Journal of Applied Engineering Research ISSN 0973-4562 Volume 12, Number 9 (2017) pp. Research ISSN 0973-4562 Volume 12, Number 9 (2017) pp. 1867-1873 ©Research India Publications. http://www.ripublication.com

- Junwon Seo , Daniel G. Linzell, Horizontally curved steel    bridge seismic vulnerability assessment, 2011 Elsevier

- C. C. SPYRAKOS &A. G. VLASSIS, EFFECT OF SOIL-STRUCTURE INTERACTION ON SEISMICALLY ISOLATED BRIDGES, Journal of Earthquake Engineering ,Volume 6, 2002 - Issue 3, https://doi.org/10.1080/13632460209350423

- Zhenyun Tang, Hua Ma, Jun Guo, and Zhenbao Li, Soil-Structure Interaction on Seismic Performance of

Long-Span Bridge Tested by Dynamic Substructuring     Method, Hindawi,Shock and Vibration, Volume 2017,    ArticleID4358081,12pages,https://doi.org/10.1155/2017/4358081

- Hong-Liang Zhang, Determination of Allowable Differential Settlement between Bridge Abutment and Approach Embankment with Five-degree-of-freedom Vehicle Model ISSN 1997-1400 Int. J. Pavement Res. Technol. 3(6):311-319

- Shawn Parks, Mijia Yang, Sivapalan Gajan and Qiang Pei, Strength-based differential tolerable settlement limits of bridges,2017 https://doi.org/10.1177/1369433217706779

- Stefanidou, S. P. , Sextos, A. G., Kotsoglou, A. N., Lesgidis, N., & Kappos, A. J. (2017). Soil-structure interaction effects in analysis of seismic fragility of bridges using an intensity-based ground motion selection procedure. Engineering Structures, 151, 366-380. https://doi.org/10.1016/j.engstruct.2017.08.033

- I.PoLam, H.Law, Soil Structure Interaction of Bridges for Seismic Analysis, MCEER-00-0008 | 09/25/2000

- Shehata Eldabie Abdel Raheem, Soil–structure interaction for seismic analysis and design of bridges, https://doi.org/10.1016/B978-0-12-823550-8.09991-1 , 2022 Elsevier